IC Layout Design of 16Bit Adder

VLSI Design System

Ahmet Deniz YILMAZ 21028656

Hacettepe University Electrical & Electronics Engg.

Abstract -This paper describes the design of an integrated circuit(IC) layout of a 16Bit adder with 2 different methods used for each 8Bits for performance comparison. First 8Bit is a ripple carry adder and second 8Bit is a carry-look ahead adder. The layout was designed by use of an open source software namely Electric

VLSI Design System.Circuit and modules tested in LTspice. The complete layout of the decoder was designed based on its schematic circuit, which consists of NOT gates, 2-input NAND gates, 3-input NAND gates, 2-input OR gates and XOR gates derived from NAND gates.

# 1.INTRODUCTION

Electronic devices have been widely used in many different fields and the size of these devices has been gradually reduced. An example of this is the mobile phone which is made smaller to enhance user’s mobility and usage time. These are the contribution of integrated circuit (IC) technology. With this technology, the modern devices have been reduced to convenient sizes. Besides that, mass production of IC has lowered the cost of production and made most electronic devices affordable. Today, an IC is smaller than a coin and can hold millions of transistors. Hence, further research in the design of IC is important to enhance the production of a more efficient and viable IC.

There are different technologies to construct integrated circuits such as bipolar integrated technology, NMOS technology and CMOS technology. In this project, CMOS technology is used. The main reason in using CMOS technology is due to its scalable high noise immunity and low power consumption. Basically, CMOS technology uses both NMOS and PMOS, which means only either one of both types of transistors will be ON at a time during the operation. Thus, CMOS IC consumes less power as power is used only when the NMOS and PMOS transistors are switching between on and off states.

The main objective of this report is design IC layout of 16Bit adder by Electric VLSI that free open source EDA system that provides service in handling IC layout, schematic drawing, textual hardware description language, and other features [1]

There are different technologies to construct integrated circuits such as bipolar integrated technology, NMOS technology and CMOS technology. In this project, CMOS technology is used. The main reason in using CMOS technology is due to its scalable high noise immunity and low power consumption. Basically, CMOS technology uses both NMOS and PMOS, which means only either one of both types of transistors will be ON at a time during the operation. Thus, CMOS IC consumes less power as power is used only when the NMOS and PMOS transistors are switching between on and off states [2].

# 2. LITERATURE REVIEW

## 2.1 Ripple Carry Adder

A ripple carry adder is a digital circuit that produces the arithmetic sum of two binary numbers. It can be constructed with full adders connected in cascaded.

-One bit adder

A one-bit full adder is a combinational circuit that forms the arithmetic sum of three.

Figure 1. One Bit Adder

In the ripple carry adder, the output is known after the carry generated by the previous stage is produced. Thus, the sum of the most significant bit is only available after the carry signal has rippled through the adder from the least significant stage to the most significant stage. As a result, the final sum and carry bits will be valid after a considerable delay.

Table 1.CMOS gate delays and areas normalized relative to an inverter.

For a 32-bit processor, the carry chain normalized delay would be 131. The ripple carry adder can get very slow when many bits need to be added. In fact, the carry chain propagation delay is the determining factor in most microprocess.

## 2.2 Carry lookahead adder (CLA)

The carry lookahead adder (CLA) solves the carry delay problem by calculating the carry signals

in advance, based on the input signals. It is based on the fact that a carry signal will be generated

in two cases:  when both bits and are 1, or  when one of the two bits is and the carry-in is.

Figure 2. 4Bit carry look ahead implementation detail

The disadvantage of CLA is that the carry logic block gets very complicated for more than 4-bits. For that reason, CLAs are usually implemented as 4-bit modules and are used in a hierarchical structure to realize adders that have multiples of 4-bits. [3]

Figure 3. 8Bit Carry-Look Ahead adder

# 3.METHODOLOGY

3.1.  Logic Gates

### 3.1.a  2-input NAND Gates

Figure 1 shows schematic diagram and icon view designed in Electric VLSI.

Exports named as Inp0, Inp1, Out and inputs colored dark-red while outputs are dark-green for easy management and readability purposes.

Figure  1. NAND2 Gate Schematic and icon view    					 Figure  2. NAND2 Gate Layout Design

We get same correct results for both schematic and layout tests shown in Figure 3 as standart truth table. Applied capacitor to eliminate to spikes caused by states transitions.

Figure  3. nand2 gate tests via Ltspice. (For both schematic and layout)

Test Spicecode for nand2 layout and schematic

* Spice Code nodes in cell cell 'Nand2Gate{lay}'

* Spice Code nodes in cell cell 'Nand2Gate{sch}'

vdd Vdd 0 DC 5

vin Inp0 0 DC 5 pulse(0 5 10f 0.5f 0.5f 20n 40n)

vin2 Inp1 0 DC 5 pulse(0 5 10f 0.5f 0.5f 40n 80n)

cload Out 0 50fF

.tran 0 100ns

.include C5_models.txt

.END

### 3.1.b  3-input NAND Gates

3-input NAND gate designed and used in XOR gate to avoid using extra transistor for applying nand gate to 3 signals since It’s also possible via 2-input NAND gates that would require more transistors.

Figure  4.  NAND3 Gate Schematic and icon view    				Figure  5. NAND3 Gate Layout Design

We get same correct results for both schematic and layout tests shown in Figure 6 as standart truth table. Applied capacitor to eliminate to spikes caused by states transitions.

Figure  6.  Nand3 gate test plot using Ltspice.(For both schematic and layout)

Test Spicecode for nand3 layout and schematic

* Spice Code nodes in cell cell 'Nand3Gate{lay}'

* Spice Code nodes in cell cell 'Nand3Gate{sch}'

vdd Vdd 0 DC 5

vin Inp0 0 DC 5 pulse(0 5 10f 0.5f 0.5f 10n 20n)

vin2 Inp1 0 DC 5 pulse(0 5 10f 0.5f 0.5f 20n 40n)

vin3 Inp2 0 DC 5 pulse(0 5 10f 0.5f 0.5f 40n 80n)

cload Out 0 50fF

.tran 0 100ns

.include C5_models.txt

.END

### 3.1.c  Inverter

Figure  7.Inverter Schematic and icon view				Figure  8. Inverter Layout Design

We get same correct results for both schematic and layout tests shown in Figure 3 as standart truth table. Applied capacitor to eliminate to spikes caused by states transitions.

Figure  9.Inverter test plot using Ltspice. (For both schematic and layout).

Test Spicecode for inverter layout and schematic

* Spice Code nodes in cell cell 'Inverter{lay}'

* Spice Code nodes in cell cell 'Inverter{sch}'

vdd Vdd 0 DC 5

vin Inp 0 DC 5 pulse(0 5 10f 0.5f 0.5f 20n 40n)

cload Out 0 50fF

.tran 0 100ns

.include C5_models.txt

.END

### 3.1.d  2-Input NOR Gate

Figure  10.Nor2 Gate Schematic and icon view			Figure  11.Nor2 Gate Layout Design

We get same correct results for both schematic and layout tests shown in Figure 12.

Figure  12.Nor2 gate test plot using Ltspice. (For both schematic and layout)

Test Spicecode for Nor2 layout and schematic

* Spice Code nodes in cell cell 'Nor2Gate{lay}'

* Spice Code nodes in cell cell 'Nor2Gate{sch}'

vdd Vdd 0 DC 5

vin Inp0 0 DC 5 pulse(0 5 10f 0.5f 0.5f 20n 40n)

vin2 Inp1 0 DC 5 pulse(0 5 10f 0.5f 0.5f 40n 80n)

cload Out 0 50fF

.tran 0 100ns

.include C5_models.txt

.END

### 3.1.e  2-Input AND Gate

Figure  13. And2 Gate Schematic and icon view			   Figure  14. And2 Gate Layout Design

We get same correct results for both schematic and layout tests shown in Figure 15 as standart truth table. Applied capacitor to eliminate to spikes caused by states transitions.

Figure  15.  And2 gate test plot using Ltspice. (For both schematic and layout)

Test Spicecode for and2 layout and schematic

* Spice Code nodes in cell cell 'And2Gate{lay}'

* Spice Code nodes in cell cell 'And2Gate{sch}'

vdd Vdd 0 DC 5

vin Inp0 0 DC 5 pulse(0 5 10f 0.5f 0.5f 20n 40n)

vin2 Inp1 0 DC 5 pulse(0 5 10f 0.5f 0.5f 40n 80n)

cload Out 0 50fF

.tran 0 100ns

.include C5_models.txt

.END

### 3.1.f  2-Input OR Gate

Figure  16. Or2 Gate Schematic and icon view			   Figure  17. Or2 Gate Layout Design

We get same correct results for both schematic and layout tests shown in Figure 18 as standart truth table. Applied capacitor to eliminate to spikes caused by states transitions.

Figure  18.  Or2 gate test plot using Ltspice. (For both schematic and layout)

Test Spicecode for or2 layout and schematic

* Spice Code nodes in cell cell 'Or2Gate{lay}'

* Spice Code nodes in cell cell 'Or2Gate{sch}'

vdd Vdd 0 DC 5

vin Inp0 0 DC 5 pulse(0 5 10f 0.5f 0.5f 20n 40n)

vin2 Inp1 0 DC 5 pulse(0 5 10f 0.5f 0.5f 40n 80n)

cload Out 0 50fF

.tran 0 100ns

.include C5_models.txt

.END

### 3.1.g  XOR Gate

Figure  19. Xor Gate Schematic and icon view

Figure  20. Xor Gate Layout Design

We get same correct results for both schematic and layout tests shown in Figure 21. as standart truth table. Applied capacitor to eliminate to spikes caused by states transitions.

Figure  21.  XOr gate test plot using Ltspice. (For both schematic and layout)

Test Spicecode for xor layout and schematic

* Spice Code nodes in cell cell 'Xor2Gate{lay}'

* Spice Code nodes in cell cell 'Xor2Gate{sch}'

vdd Vdd 0 DC 5

vin Inp0 0 DC 5 pulse(0 5 10f 0.5f 0.5f 20n 40n)

vin2 Inp1 0 DC 5 pulse(0 5 10f 0.5f 0.5f 40n 80n)

cload Out 0 50fF

.tran 0 100ns

.include C5_models.txt

.END

## 3.2.  Modules

### 3.2.a  1Bit Full Adder

1Bit Full adder( 3 bit adder) with carry in and carry out.

Figure  22. 1Bit FullAdder Schematic and icon view

Figure  23. 1Bit FullAdder Layout

Figure  24.1Bit FullAdder Test Plot

Test Spicecode for 1Bit Full Adder layout and schematic

* Spice Code nodes in cell cell ' 1BitFA{lay}'

* Spice Code nodes in cell cell '1BitFA{sch}'

vdd Vdd 0 DC 5

vin Inp0 0 DC 5 pulse(0 5 10f 0.5f 0.5f 10n 20n)

vin2 Inp1 0 DC 5 pulse(0 5 10f 0.5f 0.5f 20n 40n)

vin3 Cin 0 DC pulse(5 0 10f 0.5f 0.5f 40n 80n)

cload Sum 0 50fF

.tran 100ns

.include C5_models.txt

.END

### 3.2.b  4Bit Full Adder

4Bit Full adder from four 1Bit full adders.

Figure  25. 4Bit FullAdder Schematic and icon view

Figure  26. 4Bit FullAdder Layout

Figure  27.4Bit FullAdder Test Plot

Test Spicecode for 4Bit Adder layout and schematic

* Spice Code nodes in cell cell ' 4BitFA{lay}'

* Spice Code nodes in cell cell '4BitFA{sch}'

vdd Vdd 0 DC 5

*A Bits

vin A0 0 DC 5

vin2 A1 0 DC 5

vin3 A2 0 DC 0

vin4 A3 0 DC 5

*B Bits

vin5 B0 0 DC 0

vin6 B1 0 DC 0

vin7 B2 0 DC 5

vin8 B3 0 DC 0

cload S0 0 50fF

.tran 100ns

.include C5_models.txt

.END

### 3.2.c  8Bit Adder

8 bit adder with two 4Bit full adders.

Figure  28. 8Bit Adder Schematic and icon view

Figure  29. 8Bit Adder Layout

Figure  30. 8Bit Adder Test Plot via LTspice

Test Spicecode for  layout and schematic

* Spice Code nodes in cell cell ' 4BitFA{lay}'

* Spice Code nodes in cell cell '4BitFA{sch}'

vdd Vdd 0 DC 5

*A Bits

vin A0 0 DC 5

vin2 A1 0 DC 0

vin3 A2 0 DC 5

vin4 A3 0 DC 0

vin5 A4 0 DC 5

vin6 A5 0 DC 5

vin7 A6 0 DC 0

vin8 A7 0 DC 0

*B Bits

vin9 B0 0 DC 5

vin10 B1 0 DC 5

vin11 B2 0 DC 5

vin12 B3 0 DC 5

vin13 B4 0 DC 5

vin14 B5 0 DC 5

vin15 B6 0 DC 0

vin16 B7 0 DC 5

cload S0 0 5fF

.tran 0 100ns

.include C5_models.txt

.END

### 3.2.d  1Bit CLA Adder

1 Bit Adder for 8 bit carry look a head adder

Figure  31. 1Bit CLA Adder Schematic and icon view

Figure  32. 1Bit CLA Adder Layout

Figure  33. 8Bit Adder Test Plot via LTspice

Test Spicecode for  layout and schematic

* Spice Code nodes in cell cell ' 4BitFA{lay}'

* Spice Code nodes in cell cell '4BitFA{sch}'

vdd Vdd 0 DC 5

*A Bits

vin A0 0 DC 5

vin2 A1 0 DC 0

vin3 A2 0 DC 5

vin4 A3 0 DC 0

vin5 A4 0 DC 5

vin6 A5 0 DC 5

vin7 A6 0 DC 0

vin8 A7 0 DC 0

*B Bits

vin9 B0 0 DC 5

vin10 B1 0 DC 5

vin11 B2 0 DC 5

vin12 B3 0 DC 5

vin13 B4 0 DC 5

vin14 B5 0 DC 5

vin15 B6 0 DC 0

vin16 B7 0 DC 5

cload S0 0 5fF

.tran 0 100ns

.include C5_models.txt

.END

### 3.2.e  CLA Block

Carry look a head block for CLA network.

Figure  34. CLA Block  Schematic and icon view

Figure  35. CLA Block  Layout

Figure  36.  CLA Block Test Plot via LTspice

Test Spicecode for  layout and schematic

* Spice Code nodes in cell cell ' 1Bit CLA FA{lay}'

* Spice Code nodes in cell cell '1Bit CLA FA{sch}'

vdd Vdd 0 DC 5

vdd Vdd 0 DC 5

vin GhInp 0 DC 5 pulse(0 5 10f 0.5f 0.5f 20n 40n)

vin2 PhInp 0 DC 5 pulse(0 5 10f 0.5f 0.5f 40n 80n)

vin3 GlInp 0 DC 5 pulse(0 5 10f 0.5f 0.5f 10n 20n)

vin4 PlInp 0 DC 5 pulse(0 5 10f 0.5f 0.5f 30n 60n)

vin5 ClInp 0 DC 5 pulse(0 5 10f 0.5f 0.5f 30n 60n)

.tran 0 100ns

.include C5_models.txt

.END

### 3.2.f  8 Bit CLA Adder

Figure  37. 8Bit CLA Adder  Schematic and icon view

Figure  38. 8Bit CLA   Layout

Figure  30.  CLA Block Test Plot via LTspice

Test Spicecode for  layout and schematic

* Spice Code nodes in cell cell ' 1Bit CLA FA{lay}'

* Spice Code nodes in cell cell '1Bit CLA FA{sch}'

vdd Vdd 0 DC 5

*A Bits

vin A0 0 DC 5

vin2 A1 0 DC 5

vin3 A2 0 DC 0

vin4 A3 0 DC 5

vin5 A4 0 DC 5

vin6 A5 0 DC 0

vin7 A6 0 DC 5

vin8 A7 0 DC 5

*B Bits

vin9 B0 0 DC 0

vin10 B1 0 DC 5

vin11 B2 0 DC 0

vin12 B3 0 DC 0

vin13 B4 0 DC 5

vin14 B5 0 DC 0

vin15 B6 0 DC 0

vin16 B7 0 DC 0

cload S0 0 5fF

.tran 0 100ns

.include C5_models.txt

.END

### 3.2.g  16 Bit Mixed Adder

Figure  37. 8Bit CLA Adder  Schematic and icon view

Figure  38. 8Bit CLA   Layout

Figure  30.  16Bit adder  Test Plot via LTspice

Test Spicecode for  layout and schematic

* Spice Code nodes in cell cell ' 1Bit CLA FA{lay}'

* Spice Code nodes in cell cell '1Bit CLA FA{sch}'

vdd Vdd 0 DC 5

*A Bits

vin A0 0 DC 0

vin2 A1 0 DC 5

vin3 A2 0 DC 5

vin4 A3 0 DC 5

vin5 A4 0 DC 5

vin6 A5 0 DC 5

vin7 A6 0 DC 5

vin8 A7 0 DC 5

vin9 A8 0 DC 0

vin10 A9 0 DC 0

vin11 A10 0 DC 0

vin12 A11 0 DC 0

vin13 A12 0 DC 0

vin14 A13 0 DC 0

vin15 A14 0 DC 0

vin16 A15 0 DC 0

*B Bits

vin17 B0 0 DC 0

vin18 B1 0 DC 0

vin19 B2 0 DC 0

vin20 B3 0 DC 0

vin21 B4 0 DC 0

vin22 B5 0 DC 0

vin23 B6 0 DC 0

vin24 B7 0 DC 0

vin25 B8 0 DC 0

vin26 B9 0 DC 0

vin27 B10 0 DC 0

vin28 B11 0 DC 0

vin29 B12 0 DC 0

vin30 B13 0 DC 0

vin31 B14 0 DC 0

vin32 B15 0 DC 5

.tran 10ns 100ns

.include C5_models.txt

.END

# REFERENCES

[1]	About	Electric.(May 2013).

[2]	R. J. Baker, CMOS: Circuit Design, Layout, and Simulation, John Wiley & Sons, 2010, pp. 6-12.

[3] 	Professor F. Gebali Ripple Carry and Carry Lookahead Adders

# Figures

Figure 1. One Bit Adder	2

Figure 2. 4Bit carry look ahead implementation detail	3

Figure  1. NAND2 Gate Schematic and icon view    	     Figure  2. NAND2 Gate Layout Design	4

Figure  3. nand2 gate tests via Ltspice. (For both schematic and layout)	4

Figure  4.  NAND3 Gate Schematic and icon view    	   Figure  5. NAND3 Gate Layout Design	5

Figure  6.  Nand3 gate test plot using Ltspice.(For both schematic and layout)	6

Figure  7.Inverter Schematic and icon view	   Figure  8. Inverter Layout Design	6

Figure  9.Inverter test plot using Ltspice. (For both schematic and layout).	7

Figure  10.Nor2 Gate Schematic and icon view	  Figure  11.Nor2 Gate Layout Design	7

Figure  12.Nor2 gate test plot using Ltspice. (For both schematic and layout)	8

Figure  13. And2 Gate Schematic and icon view	     Figure  14. And2 Gate Layout Design	8

Figure  15.  And2 gate test plot using Ltspice. (For both schematic and layout)	9

Figure  16. Or2 Gate Schematic and icon view	     Figure  17. Or2 Gate Layout Design	9

Figure  18.  Or2 gate test plot using Ltspice. (For both schematic and layout)	10

Figure  19. Xor Gate Schematic and icon view	11

Figure  20. Xor Gate Layout Design	11

Figure  21.  XOr gate test plot using Ltspice. (For both schematic and layout)	12

Figure  22. 1Bit FullAdder Schematic and icon view	13

Figure  23. 1Bit FullAdder Layout	13

Figure  24.1Bit FullAdder Test Plot	14

Figure  25. 4Bit FullAdder Schematic and icon view	15

Figure  26. 4Bit FullAdder Layout	16

Figure  27.4Bit FullAdder Test Plot	17

Figure  28. 8Bit Adder Schematic and icon view	18

Figure  29. 8Bit Adder Layout	19

Figure  30. 8Bit Adder Test Plot via LTspice	20

Figure  31. 1Bit CLA Adder Schematic and icon view	21

Figure  32. 1Bit CLA Adder Layout	21

Figure  33. 8Bit Adder Test Plot via LTspice	22

Figure  34. CLA Block  Schematic and icon view	23

Figure  35. CLA Block  Layout	24

Figure  36.  CLA Block Test Plot via LTspice	24

Figure  37. 8Bit CLA Adder  Schematic and icon view	25

Figure  38. 8Bit CLA   Layout	26

Figure  30.  CLA Block Test Plot via LTspice	26

Figure  37. 8Bit CLA Adder  Schematic and icon view	27

Figure  38. 8Bit CLA   Layout	28

Figure  30.  16Bit adder  Test Plot via LTspice	29

Contents

1.INTRODUCTION	1

2. LITERATURE REVIEW	2

2.1 Ripple Carry Adder	2

2.2 Carry lookahead adder (CLA)	2

3.METHODOLOGY	4

3.1.  Logic Gates	4

3.1.a  2-input NAND Gates	4

3.1.b  3-input NAND Gates	5

3.1.c  Inverter	6

3.1.d  2-Input NOR Gate	7

3.1.e  2-Input AND Gate	8

3.1.f  2-Input OR Gate	9

3.1.g  XOR Gate	11

3.2.  Modules	13

3.2.a  1Bit Full Adder	13

3.2.b  4Bit Full Adder	15

3.2.c  8Bit Adder	18

3.2.d  1Bit CLA Adder	21

3.2.e  CLA Block	23

3.2.f  8 Bit CLA Adder	25

3.2.g  16 Bit Mixed Adder	27

REFERENCES	30

Figures	31